BEFORE STARTING ENGINE

- Preflight Insp.—Complete

- Passenger Brief—Complete

- Avionics Master Switch & Other Electrical—Off

- Circuit Breakers—Check in

- Fuel Valve—Fullest Tank

- Master Switch—On

- Seats / Belts—Adjust & Lock

- Brakes—Set

- Cowl Flaps—Open

# STARTING ENGINE

- Mixture—Rich

- Propeller—High RPM

- Throttle—Full Open

- Master Switch—On

- Fuel Boost—High 5 sec./Off

- Mixture—Idle cut-off

- Throttle—1/4 open

- Strobe Lights—On

- Propeller Area—Clear

- Ignition Switch—Start

- Mixture—Advance to Full Rich while cranking

- Throttle—1000 rpm after start

- Oil Pressure—Check

- Avionics Master—On

- Autopilot—On test mode

- Transponder—Standby-1200

- Nav. Lights—On as required

- Flaps—Retract for taxi

- Mixture—Lean as necessary

# TAXI CHECK

- Radios—Set/Check

- Taxi Area—Clear

- Brakes / Steering—Check

# BEFORE TAKEOFF

- Brakes—Set

- Seats / Belts—Check secure

- Doors—Closed & Locked

- Cowl Flaps—Open

- Trims—Set for T/O

- Autopilot—On/check

- Flight Controls—Free/Correct

- Flight Instruments—Ck/Set

- Alt. Static Source—Check

- Fuel Quantity—Check

- Fuel Valve—Fullest Tank

- Fuel Pump—Off

- Mixture—Rich

- Throttle—1700 RPM

- Suction Gauge—Check

- Engine Inst.—Check

- Ammeter—Check

- Magnetos—Ck (150 RPM
max. drop ea., 50 RPM diff.)

- Propeller—Cycle 3X

- Throttle—Idle check

- Throttle—1000 RPM or less

- Throttle Fiction—Adjust

- Radios / Avionics—Set

- Circuit Breakers—Check

- Wing Flaps—Set 10

- Mixture—Full Rich < 3000 ft.
>3000 ft. adjust for max. RPM

- Propeller—High RPM

- Heading / Compass—Check

- Lights / Pitot Heat—On as  req.

- Transponder—On / ALT.

- Time Off—Note

- Brakes—Release

# NORMAL TAKEOFF

- Wing Flaps—10 degrees

- Throttle—Full Open

- Rotate—60 KTS.

- Climb Speed—100 KTS.

# AFTER TAKEOFF

- Climb Speed—100 KTS.

- Flaps—Retract @ +200 AGL

- Climb Power—Set @ 500 AGL

- Trim—As Required

# ENROUTE CLIMB

- Airspeed—105 to 120 KTS.

- Throttle—25” / 2500 RPM

- Mixture—Lean as necessary

- Cowl Flaps—Open as req.

# CRUISE

- Power—20” to 24” / 2200 to 2400 RPM                           (75% power setting or less)

- Cowl Flaps—Open as req.

- Trims—Adjust

- Mixture—Adjust for Fuel Flow, CHT, and EGT      (CHT-- 420 Max.)
(EGT ~ 1400)

- Fuel Valve—Switch tank every 30 min. to balance fuel

- Engine Instruments--Monitor

# DESCENT

- Mixture—As Necessary

- Power—As desired

- Cowl Flaps—Closed

# BEFORE LANDING

- Fuel Valve—Fullest Tank

- Mixture—Rich

- Propeller—High RPM

- Seats Belts—Secure

- Landing Lights—On

- Cowl Flaps—Closed

# BALKED LANDING

(Go Around)

- Throttle—Full Open

- Wing Flaps—Retract to 10

- Airspeed—100 KTS.

- Cowl Flaps—Open

# NORMAL LANDING

- Airspeed—100 KTS. entering downwind (Flaps up)

- Wing Flaps—Extend (< Vfe) 10 downwind abeam touchdown point; 20 on base; 30 on final if req.

- Throttle—Maintain some power on final until flare

- Airspeed—70 to 80 KTS

- Throttle—to idle during flare

- Touchdown—Main wheels

- Rollout—Lower nose gently

- Braking—Minimum required

# AFTER LANDING

- Cowl Flaps—Open

- Wing Flaps—Retract

- Landing Light, Pitot Heat—Off

- Transponder—Standby-1200

- Trims—Set for T/O

- Mixture—Lean as necessary

# SECURING AIRPLANE

- Brakes—Set

- Electrical Equipment (except Strobes)—Off

- Transponder—Standby-1200

- Avionics Master—Off

- Mixture—Idle Cut-off

- Ignition Switch—Off

- Strobes—Off

- Interior Lights—Off

- Master Switch—Off

- Control Lock—Install

SPECIFICATIONS

#### Express Series 2000

Engine: Cont.  IO-550-N1B

#### Max. H/P: 310 HP @ 2700 RPM

Max. Cruise: 240 HP @ 2500 RPM

##### Prop:  Hartzell 3 blade CS

#### Length:  25 ft

#### Wingspan:  31 ft

#### Wing area: 130 sq ft

#### Aspect ratio:  7.4:1

Cabin width front:  46”

#### Cabin width rear:  44”

Cabin height:  42”

## Gross weight	3400 lbs max.

#### Empty weight       1902 lbs*

## Useful Load          1498 lbs

#### Fuel	    92 gal

### Payload (full fuel) 946 lbs*

*Note: see weight & balance             sheet for latest information

##### OTHER INFO

###### Est. fuel burn:	15 gal/hr

Endurance (w/ 45

minute reserve):	5.4 hr

Range @ 185 kts.

with reserve:	1000 miles

## HOT ENGINE START

- Fuel selector—On

- Throttle—Closed

- Mixture—Idle cut-off

- Fuel Boost—High for 15-20 seconds

- Perform normal start

FLOODED ENGINE

- Discontinue cranking

- Allow intake to drain

- Perform normal start

Note: Do not crank flooded engine; allow intake to drain fuel before attempting start

## AIRSPEEDS

## Vr     --  60 KTS

Vx    --  75 KTS (flaps 10)

Vy    --  98 KTS

Vno  -- 185 KTS

## Vne  -- 230 KTS

#### Va    -- 153 KTS

#### Vfe   -- 120 KTS

## Vs1   --  58 KTS

## Vso   --  55 KTS

### BEST GLIDE – 98 KTS