# Condition Inspection Checklist

Date:	__________

### empennage

left elevator

check skins for cracks

check counterweight area for cracks

inspect hinges, check for loose bushings

lubricate hinges

right elevator

check skins for cracks

check counterweight area for cracks

inspect hinges, check for loose bushings

check trim tab servo flexible cable

check trim tab skins for cracks

check trim tab hinges

check servo drive arm and trim tab horn

check elevator horn

check aft elevator push pull tube and ends

check aft elevator bellcrank

rudder

check skins for cracks

check counterweight area for cracks

check hinges, check for loose bushings

check right rudder cable to horn connect

check left  rudder cable to horn connect

horizontal stabilizer

check skins for cracks

vertical stabilizer

check skins for cracks

remove left and right inspection covers

check cables, pulleys, bellcrank

rotate both rudder pulleys 1/8 turn clockwise

check horizontal/vertical stabilizer forward junction for cracks

### wings

wing tip

check wing tip for cracks

remove wing tip

check for fuel leaks

check fuel vents

check pitot tube attachment integrity

check pitot tubing

check pitot heat operation

check electrical wiring integrity

check all visible wing fiberglass for cracks/delamination

check strobe power supply attachment integrity

check strobe light attachment integrity

remove and clean strobe light lens, check sockets

check navigation light operation

check strobe light operation

replace wingtip

aileron

check skins for cracks

check counterweight

check counterweight attachment area for cracks

check hinges

check push/pull tube connect

flap

check skins for cracks

check hinges

check push/pull tube connect

operate flap over entire range

wing external

check skins for cracks

check fuel drains for leaks

check fuel filler cap O rings

wing internal

remove all inspection covers

remove landing gear cover

check tie down supports

check aileron bell crank and push pull tube ends

check aileron torque tube and push pull tube ends

check flap push pull tube and torque tubes

check for fuel leaks

check fuel lines, fittings, plates, clamps

check landing gear attach at rib J (inboard  rib)

check landing gear attach at rib R (outboard rib)

check landing gear

check brake line

check electrical wiring and antennas

check pitot tubing

check carry through to fuselage security bolts and plates

check all visible wing fiberglass for cracks/delamination

landing gear

remove brake fairings

remove wheel pant

clean mud/debris from inside of pant, check for cracks

check gear faring for cracks

check axle attachment integrity

check brake line

check brakes for leaks/wear/cracking

check brake lining for 1/8” material

check wheel for cracks

pack wheel bearings

check wheel bearings for proper operation

check tire wear, rotate as necessary

check tire inflation, 65psi mains, 32psi nose

fuselage exterior

check entire skin for cracks with special attention to:

engine mount gusset area

nose gear support area

door hinge area

forward shear tie area

carry through area

aft shear tie area

empennage to fuselage attach area

check windows for cracks

door

check interior and exterior skins for cracks

check window for cracks

check hinge area for cracks

check hinges

clean, check, lubricate deadbolts and their receivers

check handle and latch mechanism

check strut and strut attach points

check seal

cabin (aft to front)

remove seats

remove BL162 cover

remove antenna shelf

check all visible tailcone fiberglass for cracks/delamination

check transponder antenna

check electrical wiring

check GPS antenna

check antenna ground plane

check ELT antenna

check ELT mount/attach integrity

check ELT battery replacement date: ____________

verify ELT is armed

check cables for chafing/wear

rudder tension: __________

elevator tension: ___________

check pulleys/pulley assembly

check turnbuckle integrity

check aft seat/shoulder belt connect points

check aft seat connect points

inspect all visible fiberglass in aft seat area

remove rear center console cover

check pulleys/cables

check aft shear tie fiber glass, plates and bolts

check brake line entry into fuselage

remove flap actuator cover

___ vacuum inside cabin

flap actuator system

check torque tubes

check actuator

check actuator to carry through attach bracket

check limit switches

check position sensor

check electrical wiring

observe operation over entire range

adjust digital flap indicator zero and full scale as needed

check “V” block, pulleys and cables

check turnbuckle integrity

check mid elevator bellcrank and associated mechanism

carry through, fore and aft

check for cracks/delamination especially at pins

check carry through to fuselage security bolts

check carry through pins and securing nuts

remove forward console cover

check front seat/shoulder belt connect points

inspect all visible fiberglass in front seat area

inspect pulleys/cables in forward console

check stick mechanism and all push/pull and torque tubes

check pitot tubing

check/drain pitot system water trap

check fore shear tie fiber glass, plates and bolts

remove fuel select console cover

check fuel valves

check all fuel lines/fittings

remove pulley cover at nose gear support

check fire extinguisher charge

remove nose gear support inspection cover

check pulleys and cables

check cable connection to rudder torque tubes

check rudder torque tubes

check brake pedals

check brake cylinders

check brake lines

inspect engine mount gussets, attack plates and bolts

check all visible fiberglass for cracks/delamination

check hot/cool controls for proper operation

check all wires and tubing under instrument panel

check vacuum filter (replace as necessary)

remote ELT head battery replacement date: _________

electrical loads

Turn all switches off including avionics switches.  The loads are measured by turning one switch on at time and recording the battery discharge current with that switch on.  Be sure the cowling is still attached so the taxi and landing light loads can be measured.

Turn master switch on - record initial voltage and current after turn coordinator has reached top speed.  All current readings are negative.

Initial Voltage	12.8	_____

Amps	Amps

Master	 1.5	_____

Field	 3.7	_____

Fuel Pump	 4.3	_____

Pitot Heat	10.2	_____

Nav Lights	 8.2	_____

Strobe Lights	 5.9-7.1	_____

Taxi Light	 8.7	_____

Landing Light	 8.7	_____

Cabin Lights	 1.7	_____

Panel Lights

high	 2.1	_____

low	 1.6	_____

Avionics	 2.0	_____

Audio	 2.0	_____

GPS	 3.5	_____

Com, rcv	 2.4	_____

Com, xmit	 4.0	_____

Nav	 2.6	_____

Xpndr	 2.6	_____

Autopilot	 ?	_____

Flap

down	 2.8	_____

up	 3.3	_____

Trim, hi rate	 1.7	_____

Ending voltage	12.6	_____

operate trim tab over entire range

operate at high and low rate settings

verify off setting deactivates trim tab

check cooling fan operation

check all cabin lights for operation

check all panel lights for operation

check altimeter see FAR 91.217, 91.413

perform pitot static tests as appropriate see FAR 91.217, 91.413

cowling

inspect exterior for cracking/heat damage

remove nose gear faring

remove upper and lower cowling

inspect fuselage joggle for cracks

inspect cowling interior for cracking/heat damage

inspect cowling interior for baffling chafing

inspect cowling interior for oil leaks

inspect taxi/landing lights, mounts and wiring

inspect oil access door hinges

engine mount

inspect all welds for cracks

inspect all tubes for bending/buckling

inspect lord mounts for cracks

inspect lord mount bolts

inspect firewall mount bolts

engine mechanical

check for oil leaks

check crankcase for cracks, leaks and seam bolts

check pushrod tubes

check valve covers

check rocker arm covers

check cylinder head oil return lines

check inter cylinder baffling

check baffling/cowling seal

check cylinder compression

#1: ______, #2: ______, #3: ______, #4: ______

___ check magneto timing

exhaust

check gaskets for leaks

check nuts, flange welds

check stack for weld cracks, burn through

check heat muff

note color of exhaust deposits: ____________

fuel system

check rigid fuel lines and fittings

check electric pump and mount

check mechanical pump

inspect flexible fuel lines and fittings

inspect injector lines and supports

inspect fuel injector attachments

clean injectors as needed

clean gascolator screen

clean fuel screen in throttle body

drain sump at bottom of throttle body

check pressure sending unit

check fuel pump cooling air SCAT tubing

air induction

check throttle body attachment hardware

check/clean air filter, replace as necessary; Brackett #BA16 or #BA6108

check SCAT tubing from air filter to throttle body

check alternate air door operation and light

ignition system

check condition of spark plugs

clean, gap and swap top to bottom. 0.016 to 0.019in

check harness and insulators

check p-lead connections

check magneto for oil seal leakage

check breaker felts for proper lubrication

check distributor blocks for cracks, burned areas, corrosion

check engine timing

oil system

check flexible lines/fittings

check temp sending unit

check pressure sending unit

clean/check oil cooler

change oil/filter as required

check filter element for foreign particles

check oil strainer in oil sump

check breather tubes

accessories

check vacuum pump and lines

check starter mechanical integrity

check starter electrical connections

check starter ring/gear for excessive wear

check alternator mechanical integrity

check cooling fan for cracks

check alternator belt for cracks/wear

check alternator belt for tension, 11-13ftlb new, 7-9ftlb used.

clean engine, don’t contaminate vacuum pump

electrical system

check battery box for physical integrity

check battery for physical deterioration

check battery terminals for corrosion/mechanical integrity

check all wiring for integrity, chafing

check manifold vacuum line for integrity

controls, check travel, play, check nuts, etc.

throttle

mixture

governor

hot air

cooling air (both sides)

firewall

check firewall seals

check cooling air cutoff valves

check hot air SCAT tubing

check brake hydraulic fluid level

propeller

check mounting bolts and wire tie

check torque if safety paint broken

remove spinner

inspect spinner and backing plate for cracks

inspect blades for nicks and cracks

inspect hub parts for cracks and corrosion

check hub bolts

rotate blades, check for tightness in hub pilot tube

check for grease and oil leaks

lubricate propeller per propeller service manual

nose gear

remove wheel pant

clean mud/debris from interior

check for cracks

check gear attachment to engine mount

check welds for cracking

check tubing for cracking/bending

check spindle bearing play

check fork welds

check anti shimmy damper (20lbs at end of fork)

check wheel for cracks

pack wheel bearings

check wheel bearings for proper operation

check tire wear

check tire inflation, 65psi mains, 32psi nose

operational checks

check ELT

check VOR accuracy

check glideslope accuracy

check artificial horizon

check turn and bank

check autopilot operation

check magneto RPM drop at 2100 RPM

left: _________

right: _________

check full power RPM: _________

check full power manifold pressure: _________

electrical fuel pump pressure at 0 RPM: _________

mechanical fuel pump pressure at full RPM: _________

charging voltage at 1200 RPM: _________

check idle mixture

Idle RPM: _________

Discrepancy, findings, and repair record

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

Follow-up items, “buy”, notes

_________________________ _____________________________

_________________________ _____________________________

_________________________ _____________________________

_________________________ _____________________________

_________________________ _____________________________

_________________________ _____________________________

A:N49EXinspect0900.doc