CABIN

- ARROW.

- Control Lock—Remove

- Ignition Switch—Off

- Avionics Master Switch, Electrical—Off

- Master Switch—On

- Flaps—Extend

- Fuel Quantity—Check

- Fuel Valve—Fullest Tank

- Int. / Ext. Lights—Check

- Pitot Heat—Check

- Trims—Set for Takeoff

- Master Switch—Off

- Cowl Flaps—Open

# EMPENNAGE

- Rudder Gust Lock—Remove

- Tail Tie Down—Disconnect

- Control Surfaces—Check

# RIGHT WING Trailing Edge

- Flap—Check

- Aileron—Check

# RIGHT WING

- Wing Tip—Check

- A.O.A. Ports—Clear

- Fuel Vent—Check clear

- Fuel Quantity—Check

- Fuel Cap—Secure

- Wing Tie Down—Disconnect

- Main Wheel Tire—Check

- Fuel Tank Sump—Drain

- Wheel pant secure—Check

# NOSE

- Wheel pant secure—Check

- Exhaust Pipe—Check

- Propeller / Spinner—Check

- Engine Cooling Inlet / Oil Cooler—Check clear

- Nose Tire—Check

- Engine Oil—Check*

- Gas Collator—Drain

- Exhaust Pipe—Check

- Cowl Flap—Check

# LEFT WING

- Fuel Tank Sump—Drain

- Main Wheel Tire—Check

- Wheel pant secure—Check

- Wing Tie Down—Disconnect

- Fuel Quantity—Check

- Fuel Filler Cap—Secure

- Fuel Vent—Check Clear

- Pitot Tube / Cover—Check

- Static Vents—Check Clear

- Wing Tip—Check

# LEFT WING Trailing Edge

## Aileron—Check

### Flap—Check

### Baggage Door—Lock

#### Tire pressures

## Nose wheel -- 50 psi

## Mains -- 42 psi

*Oil – add if less than 6qts;

normal operation = 7 qts.