Pilot Operating Handbook and Airplane Flight Manual

GENERAL

Section I

Table of Contents

IMPORTANT NOTICE

USE OF THIS HANDBOOK

REVISING THIS HANDBOOK

SUPPLEMENTS REVISION RECORD

GROUND TURNING CLEARANCE

THREE VIEW

DESCRIPTIVE DATA

Engines

Propeller

Fuels

Oil Capacities

Weights

Cabin and Entry Dimensions

Baggage

Specific Loadings (Max Take-Off Weioght)

GENERAL AIRSPEED TERMINOLOGY/SYMBOLS

Meterological

Power

Engine Controls/Instruments

Performance and Flight Planning

Weight and Balance

NOTES

THANK YOU.....

You have obtained what we feel is the latest state-of-the-art in the most user(and family)friendly homebuilt aircraft in the world.  Its performance is spectacular and its life almost beyond measure given reasonable care. We encourage you to become familiar with this handbook as well as the FARs that are applicable to your operation. The combination will provide you with safe and sound knowledge for operation of your personally manufactured EXPRESS.

IMPORTANT NOTICE

This  handbook must be read carefully by the owner or operator(s) of the EXPRESS in order to become familiar with its operation and to obtain all it has to offer in terms of both speed and reliability.  Herein are suggestions and recommendations to help you obtain safe performance without sacrificing outstanding economy. You are encouraged to operate your machine in accordance with and within the limits identified in this Pilot's Operating Handbook and Approved Flight Manual as well as any placards located in the airplane.

Again, another reminder- the operator should also be familiar with the Federal Aviation Regulations as applicable to the iperation and maintenance of experimental airplanes and FAR Part 91 General Operating and Flight Rules. The aircraft MUST be operated and maintained in accordance with any FAA  Airworthiness Directives which may be issued against it. It is also prudent and mandatory to operate within any established limits or Service Bulletins.

The FARs place the responsibility for the maintenance of this airplane on the owner and the operator who must ensure that all maintenance is accomplished by the owner or qualified mechanics in conformity with all airworthiness requirements wstablished for this airplane.

All limits, procedures, safety practices, time limits, servicing, and maintenance requirements contained in this handbook are considered mandatory for the continued airworthiness of this airplane, in a condition equal to that of its original manufacture.

USE OF THIS HANDBOOK

The Pilot's Operating Handbook is designed so that necessary documents may be maintained therein for the safe and efficient operation of your 4-place EXPRESS. It’s loose leaf form allows easy maintenance of updates and revisions, and is also a convenient size for storage and use within the cockpit.

The handbook is in ten basic sections in accordance with the GAMA Specification No.1, Issued 15 February 1975, Revised ! September 1984, Revision #1.

NOTE

Except as noted, all airspeeds quoted in this handbook are Indicated Airspeeds (IAS) in Knots, and assume zero instrument error.

REVISING THIS HANDBOOK

Immediately following the title page is the "Log Of Revisions"page(s). The Log of Revision pages are used for maintaining a listing of all effective pages in the handbook (except the SUPPLEMENTS section). and as a record of revisions to these pages. In the lower right corner of the outlined portion of the Log is a box containing a capital letter which denotes the issue or reissue of the handbook.  This letter may be suffixed by a number which indicates the numerical revision. When a revision to any information in the handbook is made, a new Log of Revisions will be issued. All Logs of Revisions must be retained in the handbook to provide a current record of material status until a reissue is made.

WARNING

When this handbook is used for airplane operational purposes,

it is the pilot's responsibility to maintain it in current status.

AIRPLANE FLIGHT MANUAL

SUPPLEMENTS REVISION RECORD

Section IX contains the Express Design Inc. Approved Airplane Flight Manual Supplements headed by a Log of Supplements page. On the "Log" page is a listing of the EXPRESS Approved Supplemental Equipment available for installation on the airplane. When new supplements are received or existing supplements are revised, a new "Log" page will replace the previous one, since it contains a listing of all previous approvals, plus the new approval. The supplemental material will be added to the grouping in accordance with the descriptive listing.

NOTE

Upon receipt of a new or revised supplement, compare the "Log of Revisions" page just received with the existing Log page in the manual.  Retain only the new page with the latest date on the bottom of the page and discard the old one.

DESCRIPTIVE DATA

ENGINE				Lycoming, IO-360 C1C6

PROPELLER		Hartzell, 2 Blade 7666A-2, HCE2YR Hub

FUEL						100LL (blue) Av gas

Standard System Capacities

Main, Useable				46.0 gallons (U.S.)

Auxiliary wing tanks			34.0 gallons

OIL CAPACITY

8 quarts (U.S.)

WEIGHTS

Max Take-off Weight 			2895 lbs.

Max Landing Weight			2895 lbs.

Max Baggage Comp.(4 per, full fuel)     (per Wt & bal) lbs.

CABIN AND ENTRY DIMENSIONS

Height	                                             		44.5 inches

Width - Interior, front				45 inches

Width - Interior, rear				42.75 inches

BAGGAGE

Compartment Volume				20.6 cu ft

SPECIFIC LOADINGS (Max Take-off Wt.)

Wing Loading - 200HP				21.1 lbs/sq ft

Power Loading - 200HP				14.5/lbs/hp

Useful Load - 200 HP				1195 lbs

GENERAL AIRSPEED TERMINOLOGY AND SYMBOLS

CAS	Calibrated Airspeed is the indicated speed of an 	airplane, corrected for "position error" and instrument error. 	Calibrated airspeed is equal to true airspeed

in standard atmosphere at sea level.

GS	Ground Speed is the speed of an airplane relative to the  ground.

IAS	Indicated Air Speed is the speed of an airplane as shown on the airspeed indicator when corrected for instrument error.  IAS values published in this hand-book assume zero instrument error.

KCAS 	Calibrated Airspeed expressed in "knots."

KIAS	Indicated Airspeed expressed in "knots."

TAS	True Airspeed is the airspeed of an airplane relative 	to undisturbed air which is the CAS corrected for altitude, temperature, and compressibility.

VA	Maneuvering Speed is the maximum speed at which 	application full 	available aerodynamic control will not 	overstress the airplane.

VFE	Maximum Flap Extend Speed is the highest speed 	permissible with wing flaps in a prescribed extended position.

VNE	Never Exceed Speed is the speed limit that may not be exceeded at any time.

VNO	Maximum Structural Cruising Speed is the speed that should not be exceeded except in smooth air and then 	only with caution.

VS0	Stalling Speed or the minimum steady flight speed at which the airplane is controllable in the landing configuration.

VS1	Stalling Speed or the minimum steady flight speed at which the airplane is controllable

VX	Best Angle-of -Climb Speed is the airspeed which 	delivers the greates gain of altitude in the shortest possible 	horizontal distance.  Also, speed for rotation on takeoff.

VY	Best Rate-of -Climb Speed is the airspeed which 	delivers the greates gain in altitude in the shortest possible 	time.

METEOROLOGICAL TERMINOLOGY

ISA 	International Standard Atmosphere in which

1) The air is a dry perfect gas;

2) The temperature at sea level is 15 Celsius (59 F);

3) The pressure at sea level is 29.92 in. Hg. (1013.2 mB);

4) The temperature gradient from sea level to the altitude at 	which the outside air temperature is -56.5C (-69.7F) is - 0. 	00198C (-0.003566F) per foot and zero above that altitude.

OAT 	(Outside Air Temperature) The free air static temperature, 	obtained either from in-flight temperature indicators adjusted 	for instrument error and compressibility effects, or ground 	meteorological sources.

Indicated Pressure Altitude 	The number actually read from an altimeter when the barometric subscale has been set to 	29.92 in Hg or 1013.2 millibars.

Pressure Altitude	Altitude measured from standard sea-level pressure (29.92 in Hg) by a pressure or barometric altimeter. It is the indicated pressure altitude corrected for position and instrument error. In this handbook, altimeter instrument errors are assumed to be zero. Position errors may be obtained from the Altimeter Correction Graph.

Staion Pressure	Actual atmospheric pressure at field 	elevation.

Wind	The wind velocities recorded as variables on the charts of 	this handbook are to be understood as th headwind or 	tailwind components of the reported winds.

POWER TERMINOLOGY

Take-off and Maximum Continuous	The highest power rating not 	limited by time.

Cruise Climb	The power recommended for cruise climb.

ENGINE CONTROLS/INSTRUMENTS

Throttle Control	Used to control power by introducing fuel-air 	mixture into the intake passages of the engine. Settings are 	reflected by readings on the manifold pressure gauge.

Propeller Control	This control requests the propeller to 	maintain engine/propeller rpm at a selected value by 	controlling blade angle.

Mixture Control	This control is used to set fuel flow in all 	modes of operation and cuts off fuel completely for engine 	shutdown.

EGT (Exhaust Gas Temperature)	This indicator is used to 	identify the lean and best power fuel flow for various power 	settings.

CHT (Cylinder Head Temperature)	This indicator is used to 	identify the operating temperature of the engine’s cylinders.

Tachometer	Indicates the RPM of the engine/propeller.

Propeller Governor	Regulates the RPM of the engine/propeller 	by increasing or decreasing the propeller pitch change 	mechanism in the propeller hub.

AIRPLANE PERFORMANCE AND FLIGHT PLANNING TERMINOLOGY

Climb Gradient	The ratio of the change in height during a 	portion of a climb, to the horizontal distance traversed in the 	same time interval.

Demonstrated Crosswind Velocity	The velocity of the crosswind 	component for which adequate control of the airplane duriong 	take-off and landing was actually demonstrated. The value 	shown is considered to be limiting.

MEA 	Minimum enroute IFR altitude.

Route Segment	A part of a route. Each end of that part is 	identified by:

1) a geographic location

2) a point at which a definite radio fix can be 			    established.

GPH	Gallons per hour fuel flow.

PPH	Pounds per hour fuel flow

WEIGHT AND BALANCE TERMINOLOGY

Reference Datum	An imaginary vertical plane from which all 	horizontal distances are measured for balance purposes.

Station		A location along the airplane fuselage usually given 	in terms of distance from the reference datum

Arm	The horizontal distance from the reference datum to 	the center of gravity (CG) of an item.

Moment	The product of the weight of an item multiplied by its 	arm. (Moment divided by a constant may be used to simplify 	balance calculations by reducing the number of digits.

Airplane Center of Gravity (CG) 	The point at which an 	airplane would balance if suspended. Its distance from the 	reference datum is found by dividing the total moment by the 	total weight of the airplane.

CG Arm	The arm obtained by adding the airplane’s individual 	moments and dividing the sum by the total weight.

CG Limits	The extreme center of gravity locations within which 	the airplane must be operated at a given weight.

Useable Fuel	The fuel available for flight planning purposes.

Unuseable Fuel	Fuel remaining after a runout test has been 	completed in accordance with government regulations.

Standard Empty Weight	Weigth of a standard airplane including unuseable fuel, full operating fluids and oil.

Basic Empty Weight	Standard empty weight plus any 	optional equipment.

Payload	Weight of occupants, cargo, and baggage.

Usefull Load	Difference between take-off weight, or ramp weight if 	applicable, and basic empty weight

Maximum Ramp Weight	Maximum weight approved for 	ground maneuvering. (It includes the weigth of start, run-up, 	and and taxi fuel)

Maximum Take-off Weight	Maximum weight approved for the 	start of the take-off run.

Maximum Landing weight	Maximum weigth approved for the 	landing touchdown.

Zero Fuel Weight	Weight exclusive of useable fuel.

Tare	The weights of chocks, blocks, stands, etc.used on the scales 	when weighing an airplane.

Jack Points	Points on the airplane identified by the manufacturer 	as suitable for supporting the airplane for weighing or other 	purposes.

Limitations

Section II

Table of Contents

GENERAL

AIRCRAFT OPERATING SPEEDS

POWERPLANT LIMITATIONS

OPERATING LIMITATIONS - 200 HP IO-360C1C6

OIL SPECIFICATION

PROPELLER

POWERPLANT INSTRUMENT MARKINGS

INSTRUMENT MARKINGS

WEIGHT LIMITS

CENTER OF GRAVITY LIMITS

REFERENCE DATUM

MANEUVERING LIMITS

DEMONSTRATED MANEUVERS

FLIGHT LOAD FACTOR LIMITS

TYPES OF OPERATION AND LIMITS

FUEL QUANTITIES

WINTER OPERATIONS

PLACARDS

KINDS OF OPERATIONS

MINIMUM EQUIPMENT LIST (MEL)

NOTES

GENERAL

The data approved by Express Design Inc. (EDI) and the Limitations presented herin are those established by EDI as applicable  to the Express aircraft.

This section follows the format approved by the GAMA Specification #1, and is intended to provide operating guidelines and limitations specific to the Express aircraft only. All airspeeds quoted are given conventional nomenclature, are shown in knots, calibrated airspeed, and assume zero instrument error.

AIRCRAFT OPERATING SPEEDS

Express Aircraft, General

SPEED			MARKINGS		KCAS		(mph)

Never Exceed Speed	Vne	Red Line	204		(235)

Caution, smooth air only	Yellow Arc	123-151     (142-174)

Maneuvering Speed	Va			123		(142)

Normal Op Range	Vno	Green Arc	104-151     (120-174)

Full Flap Op Range	Vfe	White Arc	50-87	       (58-100)

Clean Stall Speed	Vs			55		 (63)

Stall Speed Ldg Config	Vso			50		 (58)

POWERPLANT LIMITATIONS

OPERATING LIMITATIONS

200 HP, IO360C1C6 (LYCOMING SPECIFICATION)

T.O. & Max Continuous RPM

Full Throttle, red line				2700 RPM

Normal Operation	600 (idle) 2350 - 2450 rpm (cruise)

Cylinder Head Temperatures

Maximum				475F (246C)

Normal Operating Range	325 - 380F (163 - 193C)

Recommended			150 - 435F (65-223C)

Oil Temperatures

Maximum			240F (115C)

Desired Operating		160 - 180F (71 - 82C)

Oil Pressure

Minimum Operating (idle)			10 psig

Normal Operation			        30 - 60 psi

Maximum (starting & warm up)			100 psi

Fuel Flow

Cruise				65% = 61#/hr, 75% = 79#/hr

Fuel Pump Inlet Pressure

Maximum				+8.0 psig (0.54 atm)

Recommended				-1.0 psig (-0.07 atm)

Minimum				-2.0 psig (-0.14 atm)

Vacuum Pressure

Normal Operating Range		4.3 - 5.9 In. Hg

Fuel Grade						100 LL

Maximum Lead content				2 cc/gal

OIL SPECIFICATION

Following initial break-in of the engine it should be operated with an ashless dispesant oil (MIL-L-22851) conforming to the applicable Lycoming engine handbook. Break-in (the first 50 hours or until oil consumption has been stabilized) should be accomplished using a corosion preventative oil or straight mineral oil. Low power settings (less than 65-75%) should be avoided during the break-in period and the oil level checked frequently.

POWERPLANT INSTRUMENT MARKINGS

It is recommended that the following markings be made on the engine instrument gauges to conform to convention.

OIL TEMPERATURE

Caution (Yellow Radial)			     200 - 240F

Normal Oper. Range (Green Arc)      160 (170) to 180 (220) F	Maximum (Red radial)					240F

OIL PRESSURE

Minimum (Idle, Red radial)			       10 psi

Caution Range (Yellow arc)			10 - 30 psi

Operating Range (Green arc)			30 - 60 psi

Maximum - Cold oil (Red radial)			     100 psi

TACHOMETER

Operating Range (Green arc) 	     600 to 2750 (2450) rpm

Maximum (Red radial)			   2800 (2500) rpm

CYLINDER HEAD TEMPERATURE

Operating Range (Green arc) 			240 to 380F

Maximum (Red radial)				           460F

Recommended T.O.				           240F

MANIFOLD PRESSURE

Operating range (Green arc)		15 to 29.6 in. Hg.

Maximum (red radial)			         29.6 in. Hg.

FUEL FLOW - Lbs/Hr.

Operating Range (Green arc)			45% (85%)

41 to 77 (53 to 113)

Maximum Flow	 (Red radial)			110 (130)

VACUUM PRESSURE

Operating Range (Green arc)			4.3 to 5.9 in. Hg.

WEIGHT LIMITS, Express FG - 200 HP:

Maximum Ramp, T.O. & Landing Weight	   2895 lbs.

Maximum Baggage Weight			     250 lbs.

CENTER OF GRAVITY LIMITS

FORWARD LIMITS

The allowable Center of Gravity (CG) range is from Fuselage Station (FS) 74 to FS 84.95.

AFT LIMIT

The aft CG limit is FS 84.95 inches, and must be considered a firm limit. Loadings which place the CG further aft are dangerous and must not be accepted. A “Weight and Balance” sheet must be completed and carried in the aircraft at all times. See section VI.

REFERENCE DATUM

A Fuselage Station (FS) datum must be used to establish the aircraft weight and balance. FS 34, the forward face of the firewall, is generally a convenient location regardless of engine configuration.

MANEUVERING LIMITS

The Express Model FG is licensed as EXPERIMENAL. Spins are not approved. Maneuvers which have been flown by Express Design approved test pilots are shown in the chart below. Care must be used and smooth control inputs used at all times when performing maneuvers which involve unusual aircraft attitudes, and instruction in such maneuvers is considered prudent.

DEMONSTRATED MANEUVERS

MANEUVER			ENTRY SPEED		MAX G’S

Chandelle			160 Kts.		3.5

Lazy Eight			189 Kts.		1.0 to 1.5

Stalls (not whip stalls)					0.0 to 1.5

WARNING

Since the IO 360C1C6 engine does not have an inverted oil system extreme care must be used during low or negative G maneuvers. Lack of oil pressure may cause the propeller to go to flat pitch and engine overspeed will result. Transient oil pressure conditions near zero must be limited to less than two seconds.

NOTE

All pilots are again reminded that instruction in unusual attitudes in the Express is highly desireable. Speed buildup during maneuvers can be rapid and proper control useage throughout  the maneuver to remain within limits.

Minimum fuel in the tank under use is 20 gallons, auxilliary wing tanks should be empty. Sideslips should be limited to 30 seconds maximum and oil pressure should be monitored in accordance with the note above and fuel should be selected from the high wing during sideslips.

FLIGHT LOAD FACTOR LIMITS

Flaps up					+4.5 to -2.3 g’s

Flaps down					+2.5 to -2.0 g’s

Design Ultimate (Flaps up)			+8.8 to -4.4 g’s

MINIMUM FLIGHT CREW

Minumum crew is one (1) pilot

TYPES OF OPERATIONS AND LIMITS

The Express Model FG approved for the following types of flight when the required equipment is installed and operations are conducted as defined in this LIMITATIONS section.

1. VFR, day and night

2. IFR, day and night

WARNINGS

1. Flight operations with passengers for hire and

2. Flight into known icing  is prohibited.

FUEL QUANTITIES

Standard Wing Tank (23 gal eachwing)      46 gallons useable

Extended Wing Tank (17 gal each wing)     34 gallons useable

Total						80 gallons

FUEL MANAGEMENT

Do not take off with less than 8 gallons in the wing tanks. Fuel must be used from each wing by the pilot, maintainig left/right wing balance, KNOW YOUR SYSTEM! Many accidents involve fuel, - lack of fuel or mishandling of onboard fuel.

SEATING

This aircraft seats four adults, side by side,two front and two rear, and can be flown from either front seat. The aft seats can be be both forward or the left rear facing aft for imroved rear passenger conversations.

WINTER OPERATIONS

Winter operations are acceptable with proper oil grades for the operatng temperature.

PLACARDS

All switches, lights, controls, adjustments and circuit breakers etc. should be marked with labels identifying what the switch, control, etc. is related to and what the position selects.

Safety related items such as door opening instructions, emergency shut-offs, and seat belt/shoulder harness requirements should be placed where obvious and made clearly understandable. An example of this would be the door openning procedure. It should be placed approximately near the door handle as well as being available in the EMERGENCY  Section of this handbook (Red Tab).

An example of a switch marking is the strobe light switch. It should be labeled as “Strobe” with “on” and “off’ positions identified. Convention is up is “on” and down is “off” for electrical switches. Circuit breakers should be labeled as to their rating, i.e. “5 amp”, “3 amp”. etc.

NOTE

There are two placards that must be installed.

1. The word “EXPERIMENTAL” must be placed where it can be prominently seen upon entry into the cabin. These letters must be at least three inches high, and contrast sufficiently to be seen on entry.

2. The baggage compartment must have a placard showing the maximum baggage allowed as shown on the weigth and balance data sheet for the airplane.

In addition, the following are some recommended placards:

In front of the pilot:

Airspeed Limitations

Max Flap extend speed 98 kts

Max Full Flaps 98 kts

Near the main wing tank gauges:

Do Not Take Off With Less Than

12 Gallons in Main Tanks

Near the extended range tank fuel gauges:

17 Gallons       17 Gallons

useable          useable

If strobe equiped:

Turn Strobe OFF when taxiing in vicinity of other

aircraft or when flying in fog/clouds. Standard

position lights to be used for all night flights

Near canopy latch:

Latch Door Before Takeoff

DO NOT OPEN IN FLIGHT

Special precautions should e used during flights in/around areas of atmospheric electrical activity as in thunderstorms. This aircraft, being of composite construction, conducts electricity most readily thu such as control cables, wiring, etc., a condition to AVOID.

KINDS OF OPERATIONS

EQUIPMENT LIST

This airplane may be operated in day or night VFR or day and night IFR in the United States if the appropriate equipment is installed and operable.

You as the owner are responsible for the make-up of the Minimum Equipment List (MEL) for the airplane, and maintenance thereof prior to operation where the equipment is required. For example for a day flight, the position lights need not be operable, however a strobe or anti-collision light must be.

Minimum Equipment List

System		VFR, Day

and/or			VFR, Night

Component				IFR, Day

IFR, Night      Remarks

ELECTRICAL POWER

Alternator		0	1	1	1	         Desireable

Battery			1	1	1	1     	         Desireable

Ammeter		1	1	1	1

Voltmeter		0	0	0	0

ENGINE

Cyl Head Temp		0	0	0	0	         Desireable

Exhaust Gas Temp	0	0	0	0	         Desireable

Manifold Pressure	1	1	1	1

Oil Pressure		1	1	1	1

Oil Temperature	1	1	1	1

FLIGHT CONTROLS

Elevator Trim System	1	1	1	1

Flap Position Indicator	0	0	0	0	         Desireable

FLIGHT INSTRUMENTS

Airspeed Indicator	1	1	1	1

Altimeter		1	1	1	1

Magnetic Compass	1	1	1	1

Outside Air Temp.	0	0	0	0	         Desireable

FUEL SYSTEM

Fuel Quantity Gauges	1	1	1	1

Fuel Boost Pump	1	1	1	1	         Desireable

Fuel Selector Valve	1	1	1	1

System		VFR, Day

and/or			VFR, Night

Component				IFR, Day

IFR, Night      Remarks

ICE/RAIN EQUIP.

Pitot Heat		0	0	0	0	          Desireable

LIGHTS

Cockpit Lights		0	1	0	1	          Desireable

Landing Lights		0	0	0	0	          Desireable

Nav. (Posn) Lights	0	3	0	3

Rotating Beacon/Strobe	0	1	0	1

PNEUMATIC SYSTEM

Instrument Vacuum	0	1	1	1

Pressure Gauge	0	1	1	1

PUBLICATIONS

Pilots Oper. Hndbk. &

Airplane Flight Manual	1	1	1	1

Weight and Balance	1	1	1	1

Pilots License/Medical	1	1	1	1

Radio License		1	1	1	1

Airworthiness Cert.	1	1	1	1

Registration		1	1	1	1

Maps, VFR flight	1	1	1	1	          Desireable

Charts/Appr. Plates	0	0	1	1

RESTRAINT SYSTEM

Seat Belt/Occupant	1	1	1	1

Shoulder Harness	0	0	0	0	          Desireable

Baggage Tiedown	0	0	0	0	          Desireable

Emergency

Procedures

Section III

Table of Contents

EMERGENCY  AIRSPEEDS

ENGINE FAILURE

Rough Running Engine

ENGINE FIRE

In Flight

On Ground

EMERGENCY DESCENT

MAXIMUM GLIDE CONFIGURATION

LANDING EMERGENCIES

LANDING WITHOUT POWER

SYSTEM EMERGENCIES

Propeller Overspeed

Propeller Damage

Electrical System Failure

Landing Gear

Unlatched Door In Flight

SPINS

EMERGENCY SPEED REDUCTION

NOTES

NOTE:

All airspeeds quoted in this section are indicated airspeeds (IAS) and assume zero instrument error. The aircraft should be calibrated to determine its specific error for various configurations.

EMERGENCY AIRSPEEDS

ITEM					CONDITION

Emergency Descent				153 kts

Best Glide					90 kts (103 mph)

Landing Approach (w/o power)		80 kts (92 mph)

NOTE:

The following check-lists are presented to capture in a compact format those pilot tasks requiring rapid action. These check-lists should be kept handy for ready access by the pilot, and he should familiarize him/herself with them before flying the aircraft. Knowledge of the switch, control, gauge, etc. location quickly, even blindfolded, is highly desireable.

ENGINE FAILURE

Take-off Ground Roll/Low Altitude (Less than 700’ AGL)

Maintain control of the aircraft. If runway permits, land and attempt to stop on runway. If at low altitudes, (less than approximately 700’ AGL), pick the most suitable site within +/- 30 off the nose and set up the approach. If time permits, attempt engine start.

ITEM					CONDITION

Establish				90 kts

Declare					Emergency

Check Fuel Boost Pump 		ON

Mixture					Rich

Magnetos, cycle & return to		Both

Flaps on final				Full

In Flight

Establish 90 kt glide. Climb to reduce speed if practical, pick landing site. Attempt AIR START.

ITEM					CONDITION

Check main tank for fuel		Select tank if req’d.

Fuel boost pump			ON emergencies

Mixture					RICH

Cycle mags & return to 			BOTH

Engage starter and attempt engine start

Declare					EMERGENCY

Give posn. on freq or 			121.5

Set transponder to			7700

NOTE

If power is restored and there is any doubt as to the cause of the engine stoppage, land at the nearest airpot and determine the cause.

ROUGH RUNNING ENGINE

ITEM					CONDITION

Adjust Mixture				RICH

If no improvement carefully lean for improvement as follows:

ITEM					CONDITION

Pwr setting to approx.		2100 rpm (see Eng. Manual)

Mags, Sw to LT, BOTH, RT, then to 	BEST

Readjust mixture for			best operation

NOTE

If power is restored and there is any doubt as to the cause of the engine roughness, land at the nearest airpot and determine the cause.

ENGINE FIRE

IN FLIGHT

Determine if fire is electrical (Acrid smell)

ITEM					CONDITION

Avionics Master				OFF

Master Switch				OFF

All Radios, lights, etc.			OFF

If fire/smell clears, turn master switch ON then each item of 	equipment one at a time, waiting long enough to isolate 	cause. If no smell, assume an unknown source and:

Land as soon as possible, find and correct cause.

If fire continues:

ITEM					CONDITION

Throttle 				IDLE

Mixture					CUT-OFF

Fuel Shut-Off Valve			OFF

Fuel Boost Pump			OFF

Transponder				7700

Radio				Emergency & Location

(Use active frequency or 121.5)

Land immediately and exit the aircraft.

On ground (engine start or taxi)

ITEM					CONDITION

Throttle to				IDLE

Mixture					CUT-OFF

Radio (Twr, Unicom, etc.)		EMERGENCY &

POSITION

Master Switch				OFF

Continue cranking if during start to pull fire back into the engine. Stop and exit aircraft if taxiing. If unable to stop  fire by above means, loose dirt, sand, may be used thru cooling intakes to quench fire if no extinguisher is available.

EMERGENCY DESCENT

ITEM					CONDITION

Power to				IDLE

Propeller to				HIGH RPM

Maintain				153 kts (176 mph)

Transponder				7700 or as requested

MAXIMUM GLIDE CONFIGURATION

ITEM					CONDITION

Establish				90 kts (103 mph)

Flaps					UP

Propeller				LOW RPM

Glide distance is approximately 1.3 nm (1.5 statute miles) per 1000 feet of altitude above terain, however this may vary significantly.

LANDING EMERGENCIES

Landing without power

When landing site is selected and committed to landing the following checklist can be completed. If the terrain is harsh the gear may well absorb energy and although resulting in substancial damage to the aircraft may, in that process, afford some protection to the occupants and thereby a desireable result. When assured of reaching the landing area or committed:

ITEM					CONDITION

Seat Belts/Shoulder Harness		TIGHT

Door					LATCHED

Fuel Boost Pump			OFF

Mixture					CUT-OFF

Mags					OFF

Flaps					AS REQUIRED

Master					OFF

Airspeed			Decrease to Toughdown

Attempt to fly the aircraft and keep the wings level through the approach and landing until the aircraft comes to a complete rest. EXIT THE AIRCRAFT and remain clear until assured there is no possibility of fire.

SYSTEMS EMERGENCIES

PROPELLER OVERSPEED

The controllable pitch propeller with governor used on the Express

utilizes oil pressure from the governor to increase pitch (low rpm).

It is however dangerous to run any engine over its rated rpm and thus the method to reduce any overspeed is to immediately reduce the throttle to idle and reduce airspeed to the point where rpm control is regained. Slowly add throttle and hold airspeed well below that at which the overspeed occured. Mixture may need to be adjusted also for smooth operation. If the overspeed was significant, i.e. over 200rpm over redline, an engine inspection is called for upon landing. Engine operation for the balance of the flight ust be monitored closely.

PROPELLER DAMAGE

The propeller needs proper care. Nicks, scratches, and other types of damage require care. While the construction varies, all are highly stressed and nicks can cause stress concentrations that can result in catastrophic failures of the propeller. Refer to the manual for the propeller for proper limits of acceptable damage.

ELECTRICAL SYSTEM FAILURE

The electrical system of this aircraft is key to safe operation in today’s environment. It is required for night or IFR operations. The voltmeter is your key indicator of alternator failure which then places the entire electrical load on the battery. The battery will read approximately 12.4 volts on a full battery, and 14+ on the alternator. If you experience alternator failure:

ITEM					CONDITION

If IFR/IMC			Notify ATC Immediately

Master Switch				OFF

Avionics Master				OFF

Lights					OFF

Circuit Breakers				Check

A check of the Circuit Breakers may reveal a popped breaker indicating the source of the trouble. If so , turn all individual equipment OFF, reset the breaker and turn the Master Switch ON. If the breaker does not trip again after a few minutes , slowly turn various elements of you system ON one at a time watching for another malfunction attempting to isolate the problem.

If you feel you have isolated the problem and elect to continue the flight, remain particularly alert for another malfunction caused by/related to the first.

UNLATCHED COCKPIT DOOR IN FLIGHT

The Express door must not be opened in flight. Should a latch become disengaged, cautiously slow the aircraft to approximately 85 kts (100 mph) and attempt to relock without opening any further and/or have a passenger hold the door closed. If this fails , continue the flight at that speed and land as soon as possible.

SPINS

Intentional spins are not allowed. If a spin is entered inadvertantly, reduce the power to idle, neutralize the stick or place forward long enough to break the stall, and place the rudder full against the direction of the spin until rotation is stopped. At this point the aircraft should be recovered to level flight, with smooth positive load factor (pull-out), wings level, of no more than 4 g’s (4g’s can generally be noticed by a noticeable sagging of your jaw and cheeks) taking particular care not to reenter an accellerated stall (noticeable by sharper, more positive than normal pre-stall buffeting ) and another spin. Should this pullout buffeting occur, simply relax somewhat on the back pressure till bufeting ceases and continue the pullout. If the spin has been allowed to develop, a temporary application of power may aid in recovery

WARNING

The Express aircraft is aerodynamically very clean and thus can cosume a lot of altitude with such maneuvers.

EMERGENCY SPEED/ALTITUDE REDUCTION

Speed reductions should be accomplished by idle power settings and high rpm settings to allow the engine and propeller to assist in the speed reduction. this will cause rapid cooling of the engine but is an acceptable alternative in most cases. This procedure may be appropriate after entry into IMC by a disoriented or non-rated pilot. Even flaps can be extended if necessary, although such a procedure should be followed by an appropriate inspection.

Normal Procedures

Section IV

Table of Contents

SAFE OPERATING AIRSPEEDS

PREFLIGHT INSPECTION

Cockpit - (Checklist)

Walk Around Inspection - (Checklist)

Before Starting - (Checklist)

Starting - (Checklist)

Cold Starting

Flooding Engine - (Checklist)

Hot Strting

Pre-Taxi Checks - (Checklist)

Pre take-Off Run-up - (Checklist)

Before Take-Off - (Checklist)

Runway Checks - (Checklist)

Take-off&Climb - (Checklist)

Cruise - (Checklist)

LEANING, GENERAL

Leaning, Exhaust Gas Temperature

Leaning, Flowmeter

Leaning, Manual Mixture Control

USE OF ALTERNATE AIR

ADDITIONAL CHECKLISTS

Descent - (Checklist)

Pre-Landing - (Checklist)

Balked Landing - (Checklist)

After Landing - (Checklist)

Shutdown - (Checklist)

ABBREVIATED TAKE-OFF CHECKLIST (CIGAR)

ABBREVIATED LANDING CHECKLIST (GUMP)

HEATING AND VENTILATION

COLD WEATHER OPERATIONS

ICING CONDITIONS

NOISE

NOTES

SAFE OPERATING AIRSPEEDS

NOTE

All airspeeds in this section are indicated airspeeds (IAS) and assume zero instrument error. You should make sure your system has been correctly calibrated and account for any errors as necessary.

Max Demonstrated X-WIND Component-		20 kts

Express speeds

1) Take-off, flaps up

Rotation					65-70kts(75-80mph)

50 feet						65kts(75mph)

2) Take-off, Flaps APPROACH(1/3)

Best angle of climb					65kts(75mph)

Best rate of climb					70kts(80mph)

Cruise Climb				          90-100kts(104-115mph)

3) Landing approach

Flaps DOWN(1/3)					65kts(75mph)

Flaps UP (0 deg, faired)				70kts(80mph)

4) Balked Landing Climb				70kts(80mph)

(on establishment of positive climb)

PREFLIGHT INSPECTION

COCKPIT - (Checklist)

ITEM						 	CONDITION

1) Control Lock						REMOVE

2) Avionics master switch				OFF

3) Master Switch					ON

4) Fuel Quantity Gauge					CHECK

5) Lights (If night flight)					CHECK

6) Flaps						DOWN

7) Pitot Heat (If IFR/IMC)				CHECK

8) All Switches						OFF

WALK AROUND INSPECTION - (Checklist)

(Starting at right wing/fuselage)

ITEM							CONDITION

1) Right Flap Attach’mt (Lower side)			Secure

2) Right Aileron

Hinges						Secure

Motion						Free

Span Edges					No interference

3) Wing Tip				No damage, light secure

4) Wing Upper/Lower Surface			Sight - smooth

Inspection Plates					Secure

Leading Edge				Feel for damage

Fuel Quantities				Check, Caps secure

Fuel Drains					Strain, check for water

5) Right Main Gear

Tire						Check tread, inflation

Chocks						Removed

Brake pads						Check

Brake Line						Condition

Fairing						Secure

6) Cowling area

Tire						Check tread, inflation

Chocks						Removed

Screws/retainers					Secure

Cooloing Intakes					Clear

Oil					Check quantity, door secure

Spinner						Secure

Blades    						No Damage

WARNING

Always assume the propeller is “Hot” and the engine ready to start when handling the propeller regardless of mag position.

CAUTION

See propeller manufacturer’s instructions for nick and damage treatments and limitations. Damaged propellers are dangerous and can result in catastrophic failures.

8) Left Main Gear

Tire						Check tread, inflation

Chocks						Removed

Brake pads						Check

Brake Line						Condition

Fairing						Secure

9) Wing Upper/Lower Surface			Sight - smooth

Inspection Plates					Secure

Leading Edge				Feel for damage

Fuel Quantities				Check, Caps secure

Fuel Drains					Strain, check for water

10) Wing Tip				No damage, light secure

11) Left Aileron

Hinges						Secure

Motion						Free

Span Edges					No interference

12) Left Flap Attach’mt (Lower side)			Secure

13) Tail Assembly

Horizontal Stabilizer			No leading edge damage

Vertical Stabilizer			No leading edge damage

Elevator/Rudder					Free motion

Hinges						Secure

Rudder Cables					Secure

BEFORE STARTING - (Checklist)

1) Baggage			Stowed, loose items SECURED

2) Seat Belts				Adjusted and SECURED

3) Brakes						SET

4) Circuit Breakers				Checked and IN

5) Master Switch					OFF

6) Avionics Master Switch				OFF

7) Avionics Switches 					OFF

8) Door							LATCHED

STARTING - (Checklist)

1) Master Switch					ON

2) Fuel Quantity					CHECK

3) Cowl Flaps						OPEN

4) Mixture						FULL RICH

5) Throttle						1/4 OPEN

6) Boost Pump						ON 4 SEC

7) Boost Pump						OFF

8) Clear Propeller			LOOK and call “CLEAR”

9) Starter						ENGAGE

10) On Start						1000 RPM

11) Oil Pressure					CHECK

(Shut down if inadequate)

12) Flaps						UP

13) Alternator						ON

14) Avionics Master					ON

15) Avionics						ON

COLD STARTING

Cold starts are similar, except more fuel may be required, implying longer boost pump operation. For temperatures below 20F, pre-heating is recommended.

FLOODED ENGINE - (Starting Checklist)

1) Mixture					CUT-OFF

2) Propeller					HIGH RPM

3) Throttle					1/2 OPEN

4) Crank engine

5) Upon start, throttle				IDLE

6) Mixture					RICH

WARNING

Should a backfire occur during any start, continue cranking to draw any fire back into the engine. If backfiring continues or fire starts, exit the airplane and use fire extinguisher to put out fire.

PRE-TAXI CHECKS - (Checklist)

1) Taxi Clearance				Obtain and read back

2) Clear aft area prior to power				Clear

3) Brakes						CHECK

PRE TAKE-OFF RUN-UP - (Checklist)

1) Door							LOCKED

2) Area							CLEAR

3) Brakes						SET

4) CHT/Oil Temp					GREEN

5) Throttle						1700 RPM

6) Propeller						CYCLE twice

7) Mags				CHECK for 50 RPM drop

8) Instrument Vacuum			CHECK 4.5-5.5 in. Hg.

9) Throttle					IDLE, 1000 RPM

BEFORE TAKE-OFF - (Checklist)

1) Door							LOCKED

2) Seat belts						SECURE

3) Instruments						CHECK

4) Fuel Quantity					CHECK

5) Oil Temp./Press				GREEN/CHECK

6) Breakers							IN

7) Master switch/Avionics master				ON

8) Radios							SET

9) Transponder						STANDBY

10) Propeller							IN

11) Mixture						FULL RICH

12) Boost Pump						ON

13) Trim					SET TAKE-OFF

14) Flaps					SET 15 DOWN

15) Controls					Check FREE

16) Cowl Flaps					Check OPEN

RUNWAY CHECKS (After Cleared) (Checklist)

1) Strobes							ON

2) Transponder						ON ALT

3) Approach and Take-off area				CLEAR

4) Clearance from Tower				RECEIVED

5) Runway and approach				CLEAR

6) Time							NOTE

TAKE-OFF AND CLIMB - (Checklist)

1) Take-off Power				2700 RPM

2) Oil Temp (Green)				120F minimum

3) Cylinder Head Temp (Green)			140F minimum

4) Check engine instruments

5) Check Flight Instruments

6) Rotate					65 kts (75 mph)

7) Initial climb					90 kts (104 mph)

8) At  700 feet AGL				FLAPS UP

9) Reduce power				25 in., 2500 RPM

10) Mixture					LEAN for Climb*

11) Cylinder Head Temp (Green)		430F Maximum12) 12) Oil Temp					240F Maximum

13) Boost Pump				Check Press., OFF

CRUISE - (Checklist)

1) Throttle						SET

2) Propeller					SET (Max 2500 RPM)

3) Mixture						LEAN*

4) Cowl Flaps						CLOSE

5) Fuel tanks						MONITOR

* LEANING RULES

A. Never exceed the maximum cylinder head temperatures

B. For maximum service life, CHT’s should be maintained below 435F (224C) during high performance cruise operations and below 400F (205C) for economy cruise powers.

C. Maintain “Full Rich” for Take-off, climb, and cruise power settings of above 75% power. For Take-Off from high altitude airports, if engine roughness is noted, lean only enough to obtain smooth operation. Be alert for temperature rise. This is most likely to occur at altitudes over 5000 feet, and may require opening of cowl flaps.

D. Always return to full rich before increasing power settings.

E. Operate the engine at maximum power mixture for performance cruise powers and at best economy mixture for economy cruise power.

F. During let-down flight operations it may be necessary to manually enrichen fuel/air mixture.

G. Changes to cruise altitude and/or power settings require the mixture to be reset.

LEANING, EXHAUST GAS TEMPERATURE

A. Maximum power cruise - (Approx 75% power) - 150F on rich side of peak EGT for best power. Monitor cylinder head temperature.

B. Best economy cruise - (Approx 75% power or below) - Operate at 50F lean of peak EGT.

LEANING, FLOWMETER

Lean to the applicable fuel-flow tables.

LEANING, MANUAL MIXTURE CONTROL

1) Slowly move mixture control from “Full Rich” towards lean position.

2) Continue leaninguntil slight loss of power or onset of roughness.

3) Enrich until roughness or loss of power cease.

ADDITIONAL CHECKLISTS

DESCENT - (Checklist)

1) Master Switch 					ON

2) Mags						BOTH

3) Fuel Tank					FULLEST TANK

4) Fuel Boost Pump			ON within 1000 feet AGL

5) Altimeter						SET

6) Mixture			Enrichen thru descent to FULL RICH

7) Power						As required

8) CHT’s					Greater than 180F

PRE-LANDING - (Checklist)

1) Seat Belts						SECURE

2) Fuel Tank						Fullest

3) Mixture						RICH

4) Flaps						FULL

5) Propeller						IN

6) Brakes						CHECK

7) Establish 				NORMAL APPROACH

8) Fuel Boost Pump					ON

BALKED LANDING - (Checklist)

1) Throttle						FULL

2) Airspeed					85 kts(98 mph)

3) Establish climb

4) Flaps						RETRACT

(After leaving runway)

1) Fuel boost pump					OFF

2) Flaps						UP

3) Strobes						OFF

4) Transponder						OFF

5) Lights					As required

6) Trim 						Reset to Take-Off

7) Time							Note

SHUTDOWN - (Checklist)

1) Radios						OFF

2) Avionics Master					OFF

3) Throttle						1200 RPM

4) Mixture					IDLE CUT-OFF

5) Mags						OFF

6) Lights						OFF

7) Master Switch					OFF

8) Control Lock						INSTALLED

9) Chocks/Tiedown					SECURE

10) Brakes						CHECK

ABBREVIATED TAKE-OFF CHECKLIST (CIGAR)

Controls						FREE

Instruments

Breakers					IN

Altimeter					SET

Directional Gyro				SET

Radios						SET

Engine Instruments				GREEN

Gas

Boost Pump					ON

Fuel Pressure					OK

Fuel Tanks					FULL

Mixture						RICH

Attitude

Door						SECURE

Seat Belts					SECURE

Flaps						SET

Trim						SET

Run-up

Brakes						SET

Nose wheel					Straight

Mag check			1700 RPM, 150 max drop

Propeller				TWO CYCLES

Oil Pressure					GREEN

ABBREVIATED LANDING CHECK-LIST - (GUMP)

Gas

Tank						FULLEST

Boost Pump					ON

Fuel Pressure					GOOD

Under Carriage

Brakes						CHECK

Flaps					SET 1/3 at 100 kts

Flaps					FULL at 90 kts

Mixture

Mixture Control				RICH

Prop

Propeller Control				IN

Heating and Ventilation

Cooling Air

To avoid any possibility of ingesting exhaust as part of the cooling air, and to avoid the need for additional surface obstructions, the cooling air intakes are included in the upper cowling, and accept air in the front top area of the engine cooloing air intake area. This also provides cooling air flow whenever the engine is running. This air is available to a pair of adjustable jets in the instrument panel, and another pair in the rear of the cabin for the back passengers. By operating the two side controls under the panel, cool air can also be routed into the front foot area.

In case of fire, the cooling air may be turned off by pulling the Fire/Air cutoff knob under the center of the panel.

Heating

Cabin  heat is generated by a heat muff surrounding the front exhaust crossover. From there it passes through the firewall via a rotary control valve, which can shut off the heat flow, route it to the floor vents, and route it to the defrost vents, which are located in the glare shield. A control mounted at the bottom of the instrument panel. In addition, two sidewall mounted controls select warm or cold air for the rear of the cabin. If warm air is selected for the rear, then the outside cold air intakes must be set to cut-off. As an additional feature, if the heat is shut off with the heat control valve, then cold air may be routed to the floor outlets by setting the rear air selectors to warm air.

COLD WEATHER OPERATIONS

Pre-flight inspection of the Express aircraft is particurly important in cold weather, given the particularly slick design and laminar flow airfoils. Be sure that there are no frost or ice deposits, and that all the control surfaces are free to move properly.

Very cold temperature require special attention to engine operations. The general recommendation is to pre-heat the engine below 20 Deg. F, and avoid flight engine startups below 10 Deg. F.

Warm up the engine with particular care and attention paid to oil temperature being in the green before take-off. Also, verify the responsiveness of the engine to throttle inputs.

Also, cycle the propeller and ensure it is responding properly.

Flight in icing conditions is prohibited.

Performance

Section V

Table of Contents

INTRODUCTION TO PERFORMANCE

ALTIMETER CORRECTIONS

AIRSPEED SYSTEM CALIBRATION

STALL SPEEDS

TAKEOFF DISTANCES

RATE OF CLIMB

CRUISE SPEEDS

OTHER CHARTS

Cruise speed data

International standard atmosphere

Temperture conversion

Engine chart

SUMMARY

Weight and Balance

Section VI

Table of Contents

GENERAL

LOADING CHECKS

CARGO

DEFINITIONS

WEIGHT AND BALANCE REPORT

WEIGHT AND BALANCE RECORD